Best-X-DC With Tim-X

CODE 8100M

The new BEST-X-DC with Tim-X, includes the palmtop TIM -X for a total remote control of all exposure parameters, allowing for a more compact and straightforward installation and operation.

The exclusive market leading 0.3 mm focal spot tube, grants a more detailed X-Ray image with any type of film, sensors and phosphor plates, as well as reducing to the minimum both exposure time and emitted X-ray dose. 
It is lightweight and compact, its tube head (only 3.5 kg) allows for easy maneuverability, with immediate, precise and vibration-free positioning. 
TIM-X is a state-of-the-art wireless radio-frequency palmtop which allows full control of the X-Ray unit in total safety, ensuring ease of handling. 
Remote management of kilovolt [kV], milliamperes [mA], and exposure time [ms], makes the Best X-DC TIM-X a unique device of its kind. 
Exposure times are pre-programmed (as per R10 scale) and these may be further customized to suit specific radiographic requirements. The constant DC generated X-Ray output, significantly reduces patient dose. Further operator and patient safety, is provided by the system’s exclusive thermos witch, that stops X-Ray exposure emission, in the unlikely event of failure of the exposure timer.

| Technical Data | BEST-X-DC With Tim-X |
| --- | --- |
| Classification | Class II Electro-medical Apparatus with type-B part applied |
| Head | Constant potential radiographic managed by microprocessor |
| Rated electrical power | 0,800 [kW] |
| High frequency technology | 100 [kHz] |
| Focal spot | 0,3 mm (IEC 336) |
| Absorption | 6 [A] |
| Tube | OX/70-3 |
| Ø of long cone at the end of spacer | < 6 [cm] |
| Total filtration | 2 [mm] Al eq. |
| Selectable anodic current | 4 – 6,5 [mA] |
| Selectable voltage to x-ray tube | 60 – 70 [kV] |
| Dispersed radiation | ≤ 0,25 [mGy/h] at 1 meter from focus |
| Exposure time | 20 ÷ 1000 [ms] scale R10 |
| Intermittent operation Power supply | 230 [V] ± 10% |
| Intermittent operation | 1 [s] exposure 30 [s] reset max |
| Distance | F.F.D. 20 [cm] |
| DAP [mGy*cm2] | For each exposure the DAP value is indicated |
| Deadman logic control | Wireless |

Conformity X-Ray units are manufactured according to Directive 93/42/EEC